Face Verification on LFW

Results Report — Milestone 2

Model: FaceNet  |  Metric: Cosine Similarity  |  Dataset: LFW

# 1. Overview

This report evaluates a FaceNet-based verification pipeline across five iterative runs. Each run introduces specific data or preprocessing improvements to optimize performance. To ensure a rigorous evaluation, identities are split before pair generation, preventing any individual from appearing in multiple splits and eliminating identity leakage. All runs use cosine similarity, with decision thresholds optimized for F1-score on the validation set before being applied to the held-out test split.

# 2. Run Summary

The table below summarizes all five runs. Test F1, TPR, and FPR are reported at the threshold selected on the validation split.

| Run | Change | Threshold | F1 | TPR | FPR |
| --- | --- | --- | --- | --- | --- |
| Run 001 | Baseline | 0.35 | 97.02% | 97.80% | 3.80% |
| Run 002 | Uniform Identity Weighting | 0.35 | 97.02% | 97.80% | 3.80% |
| Run 003 | Deduplication | 0.32 | 94.59% | 96.20% | 7.20% |
| Run 004 | Identity Cap | 0.35 | 95.77% | 96.35% | 3.20% |
| Run 005 | Center-Crop Preprocessing | 0.43 | 98.93% | 98.78% | 0.60% |

Table 1. Test-set results for all five runs. Bold TPR marks the best result.

Performance stabilized early, with Runs 001 and 002 producing identical results F1=97.02% due to a consistent pair distribution. Run 003 (deduplication) and Run 004 (identity capping) intentionally increased evaluation difficulty by removing redundant pairs and limiting the influence of high-frequency celebrities. This led to a more diverse, realistic test set. Run 005 introduced center-crop preprocessing, which yielded peak performance: F1 98.93%, TPR 98.78%, and a significantly reduced FPR of 0.6%.

# 3. ROC Curves and Threshold Trade-off

The following charts show the validation ROC curve and the threshold trade-off (false accepts vs. false rejects) for Run 001 (baseline) and Run 005 (best run). The ROC curves for Runs 002–004 are materially identical to Run 001 and are omitted for brevity.

## Run 001 Baseline and Run 005 Final

Figure 1. ROC curve (left) and threshold trade-off (right). Optimal threshold T=0.35 and 0.43 chosen by F1 on validation set.

# 4. Confusion Matrices

Confusion matrices are shown for Run 001 (baseline) and Run 005 (final model) side by side. The improvement from Run 001 to Run 005 is visible in the substantial reduction of both false positives (19 → 3) and false negatives (11 → 4).

Figure 2. Left: Run 001 (Baseline) - F1 0.9702, FPR 3.8%. Right: Run 005 (Center-Crop) - F1 0.9893, FPR 0.6%.

# 5. Error Analysis

Error analysis was performed on the validation split using two slices: false negatives (same person predicted as different) and boundary false positives (different people predicted as same, within 0.05 of the threshold).

## Slice 1 — False Negatives (Same Person, Low Similarity)

These are same-identity pairs where the cosine similarity fell below the threshold. The pairs are sorted ascending by score; the worst misses appear first. The two images of the same person differ substantially in pose, lighting, expression, or have significant facial obstruction pushing the embeddings apart in cosine space.

Figure 3. False negative pairs (label=1, pred=0). Score S and delta D from threshold shown below each pair. Low scores indicate high pose/lighting divergence between the two images of the same person.

## Slice 2 — Boundary False Positives (Different People, Near Threshold)

These are different-identity pairs the model was nearly certain about but got wrong, with scores within 0.05 above the threshold. The pairs shown below involve faces that are visually similar in age, hair colour, and expression — placing them in an overlapping region of FaceNet's embedding space.

Figure 4. Boundary false positive pairs (label=0, pred=1, score ≤ threshold+0.05). Both pairs show older Caucasian males with similar hair and face structure — a systematic failure mode for embedding-based verification without a dedicated face detector.

# 6. Conclusion

The final model (Run 005) achieves F1 0.9893, TPR 0.9878, and FPR 0.006 on the held-out test set. This represents a meaningful improvement over the baseline (Run 001: F1 0.9702, FPR 3.8%), driven by two complementary changes: identity-capped pair construction (Run 004) which ensured evaluation diversity across a wide population rather than a small set of prolific celebrities, and center-crop preprocessing (Run 005) which gave FaceNet a more consistent face-centred input field.

The error analysis confirms that remaining failures fall into two interpretable categories: same-identity pairs with high intra-class variation (pose and lighting), and cross-identity pairs involving visually similar faces near the decision boundary. Both failure modes suggest that a dedicated face detector and alignment step (e.g. MTCNN) would be the highest-leverage next improvement.